Purchase Specifications for a 
SL-BLK + Solar Bridge Lighting Kit

Overview

This specification is for a SL– BLK+ Bridge Lighting Kit.

SL-BRK is an LED bridge lighting kit specifically designed to clearly mark bridges and structures extending over navigable waterways. The kit includes large power supply and sectored lantern (a range of extension arms and mounting assemblies are also available on request).

The manufacturer will be able to provide the option of a self-contained, solar powered assembly housed in a weatherproof enclosure including solar array, sealed battery and charge controller. The lantern power supplies are available individually and are suitable for powering up to 12 sectored bridge light heads (SL-BR).

The lanterns shall be delivered ready to install. The only assembly required will be activation of each individual lantern and optional mounting accessories.

1.0 Light Characteristics

The light is available in two configurations, nominal 12VDC or optional universal AC (110-240VAC)

The lantern output shall be available in Red, Green, other colours available on request.

The lantern shall have a visible range of up to 4NM.

The lantern shall have a horizontal output of 360 or 180 degrees.

The lantern shall have a vertical divergence of nine (9) degrees.

The lantern shall have Steady-on (standard). Other factory set flash patterns available on request recommended user-adjustable flash characteristics.

2.0 Electrical Characteristics

The lantern shall have integrated circuit protection.

The DC lantern shall have a nominal voltage of 12–48 V.

The  optional AC lantern shall have a nominal voltage of 110–240 V.

The lantern shall have an operating temperature range between -40 to 80°C.

3.0 Physical Characteristics

The body of the lantern shall be manufactured from UV-stabilised LEXAN® polycarbonate.

The lantern lens shall be manufactured from UV-stabilised LEXAN® polycarbonate.

The lantern shall have a lens diameter of 107mm (41¼ inches).

The lantern shall have a lens design using LED optics.

The lantern shall have a mounting pattern using 4 hole brass insert pattern.

The lantern shall have a height of up to 105mm (4 inches) dependent on model.

The lantern shall have a width of 107mm (4¼ inches).

The lantern shall have a mass from 1.2kg (2⅝lbs) dependent on model.

The lantern shall have a life expectancy of up to 12 years.

4.0 Options

The lantern shall be offered with the following options available from the manufacturer:

- GSM monitoring and control system

- GPS synchronisation (DC Only)

- Hard-wire synchronisation (DC Only)

- AIS remote monitoring

- Additional cable

- Solar/battery systems

5.0 Optional Solar/Battery Configurations

The manufacturer will be able to provide the option of a self-contained, solar powered assembly housed in a weatherproof enclosure including solar array, sealed battery and charge controller. The solar module has an adjustable mount which can be angled to maximise solar collection. The light head is attached to the power supply using heavy-duty cabling enabling the lights to be mounted in shaded areas while the solar panel can be positioned in full sunlight.

Type 1

- 10 watt multicrystalline solar module

- 10 watt for powering 1-2 Sealite sectored bridge light heads

- 7.5Ah 12v SLA (Sealed Lead Acid) battery

- Housed in a 7-stage powder-coated aluminium enclosure

- Mounting will be 50mm OD base pattern.

- Height: from 288mm (11⅜ inches).

- Width: 370mm (14½ inches)

- Mass: from 6.8kg (15 lbs)

Type 2

- 20 watt multicrystalline solar module

- 20 watt for powering 1-6 Sealite sectored bridge light heads

- 26Ah 12v SLA (Sealed Lead Acid) battery

- Housed in a 7-stage powder-coated aluminium enclosure

- Mounting will be 20mm OD base pattern

- Height: from 460mm (18⅛ inches)

- Width: 656mm (25¾ inches)

- Mass: from 29.5kg (65 lbs)

Type 3

- 40 watt multicrystalline solar module

- 40 watt for powering 6-12 sectored bridge light heads

- 55Ah 12v SLA (Sealed Lead Acid) battery

- Housed in a 7-stage powder-coated aluminium enclosure

- Mounting will be 50mm OD base pattern

- Height: from 460mm (18⅛ inches)

- Width: 656mm (25¾ inches)

- Mass: from 44kg (97 lbs)

6.0 Certifications

The lantern shall be IP68 waterproof.

The lantern shall meet CE EN61000-6-3:2001. EN61000-6-1:2001. EN55022. EN61000-4-2:1995. EN61000-4-3:2002

The lantern shall meet IALA recommendation E-200-1 Signal colours

The manufacturer shall be ISO9001:2008 certified.

7.0 Warranty

- The lantern shall have a three (3) year warranty, excluding battery which will have a warranty of one (1) year.